# Lego EV2 Large Motor Notes

## Cable Color Coding (if applicable):

- Red: VCC (Power)

- Black: GND (Ground)

- Yellow: PWM (Speed Control)

- Green: Direction Control

## ESP32 Pinout:

- GPIO 12: PWM signal to control motor speed.

- GPIO 13: Direction control (forward/reverse).

- GND: Common ground with the motor driver.

The EV3 Large Servo Motor uses a 6-wire cable because it includes an encoder for precise motor control. However, the L298N motor driver only supports simple DC motor control (without encoder feedback).

### Understanding the EV3 Large Motor Wires

The 6 wires inside the cable are typically color-coded as follows:

| Wire Color | Function | L298N Connection |
| --- | --- | --- |
| Red | Motor Power A | Motor Output A (OUT1) |
| Black | Motor Power B | Motor Output B (OUT2) |
| Green | Encoder GND | Not used |
| White | Encoder VCC | Not used |
| Blue | Encoder Channel A | Not used |
| Yellow | Encoder Channel B | Not used |

Since the L298N cannot read the encoder, we will only connect the red and black wires to drive the motor like a regular DC motor.

I discovered that White and Black if connected directly to 9V move the motor. So the above table the Red and Black are Black and White.

| LEGO Motor Pin | LEGO Cable Wire Color* | BeagleBone Blue Connector/Pin | Description |
| --- | --- | --- | --- |
| 1 | white | M1/2 | Motor – |
| 2 | black | M1/1 | Motor + |
| 3 | red | E1/1 | GND |
| 4 | green | E1/2 | 3.3V |
| 5 | yellow | E1/3 | quadrature encoder A |
| 6 | blue | E1/4 | quadrature encoder B |

### Wiring EV3 Large Motor to L298N & ESP32

#### L298N to ESP32

- L298N IN1 → ESP32 GPIO 18

- L298N IN2 → ESP32 GPIO 19

- L298N ENA → ESP32 GPIO 5 (for PWM speed control)

- L298N GND → ESP32 GND

- L298N VCC → External Power Source (e.g., 9V or 12V battery pack)

#### EV3 Large Motor to L298N

- Motor Red Wire → L298N OUT1

- Motor Black Wire → L298N OUT2

### MicroPython Code for ESP32 (L298D)

This code allows you to control the speed and direction of the EV3 Large Motor using PWM signals.

from machine import Pin, PWM

import time

# Define motor control pins

IN1 = Pin(18, Pin.OUT)  # Direction 1

IN2 = Pin(19, Pin.OUT)  # Direction 2

ENA = PWM(Pin(5))       # Enable (PWM for speed control)

ENA.freq(1000)          # Set PWM frequency to 1kHz

def motor_forward(speed):

ENA.duty(speed)  # Set speed (0-1023)

IN1.value(1)

IN2.value(0)

def motor_backward(speed):

ENA.duty(speed)  # Set speed (0-1023)

IN1.value(0)

IN2.value(1)

def motor_stop():

ENA.duty(0)  # Stop motor

IN1.value(0)

IN2.value(0)

# Test motor

motor_forward(512)  # 50% speed forward

time.sleep(2)

motor_backward(512)  # 50% speed backward

time.sleep(2)

motor_stop()  # Stop motor

### Important Notes

- The L298N needs an external power supply (9V or 12V recommended).

- You will lose encoder feedback, meaning you cannot get precise position control.

- If you want precise movement control, you need an H-bridge with encoder support like the DRV8833 and an ESP32 library to read the encoder pulses.

Let me know if you want to read the encoder or need a KiCad schematic for this setup!

## How to use ENA and ENB

Yes, the L298N motor driver has two enable pins:

- ENA (Enable A) → Controls Motor A (OUT1 & OUT2)

- ENB (Enable B) → Controls Motor B (OUT3 & OUT4)

Since you are using only one motor (EV3 Large Motor), you will connect ENA to your ESP32 for speed control.

### What Do the Switches on ENA and ENB Do?

The small jumpers (switches) on ENA and ENB determine if PWM control is enabled:

- Jumper ON → The motor runs at full speed (PWM is disabled).

- Jumper OFF → You can control the speed using PWM from the ESP32.

Since we want to control the speed, remove the jumper on ENA and connect it to the ESP32.

### Updated Wiring for EV3 Large Motor and L298N

#### ESP32 to L298N

| L298N Pin | ESP32 Pin | Function |
| --- | --- | --- |
| IN1 (Motor A) | GPIO 18 | Direction 1 |
| IN2 (Motor A) | GPIO 19 | Direction 2 |
| ENA (Enable A) | GPIO 5 (PWM) | Speed Control |
| GND | ESP32 GND | Common Ground |
| VCC | External Power (9V-12V) | Motor Power |

#### EV3 Large Motor to L298N

| EV3 Wire Color | L298N Pin |
| --- | --- |
| Red (Motor A) | OUT1 |
| Black (Motor A) | OUT2 |

Important:

- Remove the ENA jumper so the ESP32 can control the motor speed via PWM.

- The ENB jumper can stay ON since we are not using Motor B.

### MicroPython Code for ESP32 with L298N

This code allows you to control both direction and speed of the EV3 Large Motor.

from machine import Pin, PWM

import time

# Define motor control pins

IN1 = Pin(18, Pin.OUT)  # Direction 1

IN2 = Pin(19, Pin.OUT)  # Direction 2

ENA = PWM(Pin(5))       # Enable (PWM for speed control)

ENA.freq(1000)          # Set PWM frequency to 1kHz

def motor_forward(speed):

ENA.duty(speed)  # Set speed (0-1023)

IN1.value(1)

IN2.value(0)

def motor_backward(speed):

ENA.duty(speed)  # Set speed (0-1023)

IN1.value(0)

IN2.value(1)

def motor_stop():

ENA.duty(0)  # Stop motor

IN1.value(0)

IN2.value(0)

# Test motor

motor_forward(512)  # 50% speed forward

time.sleep(2)

motor_backward(512)  # 50% speed backward

time.sleep(2)

motor_stop()  # Stop motor

### Summary

- Remove the jumper on ENA to enable speed control.

- Connect ENA to ESP32 (GPIO 5) for PWM speed control.

- Motor direction is controlled via IN1 and IN2.

- Use a 9V or 12V power source for the L298N.

Let me know if you need help with encoder feedback or a KiCad schematic!

## Input Voltage

The L298N motor driver has two power inputs:

- 12V (or 9V-12V input) → Main power for the motors

- 5V (regulated output) → Power for the L298N logic circuit

### How to Power the L298N Correctly

#### Which Power Input to Use?

- Use the 12V input (or 9V if you have a lower-voltage power supply).

- The L298N has a built-in 5V voltage regulator that powers the logic circuit.

- The 5V pin is an output, so you can use it to power the ESP32 if needed.

#### What Does the Jumper Above 12V/5V Do?

- This is the 5V regulator jumper that controls whether the L298N uses its internal 5V regulator.

- If the jumper is ON (default):

  - The L298N’s internal 5V regulator is active.

  - The L298N takes 12V from the input and converts it to 5V for its logic circuits.

  - The 5V pin will act as an output, and you can use it to power other components.

- If the jumper is OFF:

  - The internal 5V regulator is disabled.

  - You must provide an external 5V supply to the 5V pin for the logic circuit.

#### Recommended Jumper Setting

- Leave the jumper ON if you are using 9V or 12V power for the L298N.

- Remove the jumper if you want to power the ESP32 separately with its own 5V source and avoid interference.

### Final Wiring: ESP32 + L298N + EV3 Large Motor

| L298N Pin | ESP32 Pin | Function |
| --- | --- | --- |
| IN1 | GPIO 18 | Direction 1 |
| IN2 | GPIO 19 | Direction 2 |
| ENA | GPIO 5 (PWM) | Speed Control |
| GND | ESP32 GND | Common Ground |
| 12V | External 9V-12V Power | Motor Power |
| 5V | (Leave unconnected, or use it to power ESP32 if needed) |  |

| EV3 Wire Color | L298N Pin |
| --- | --- |
| Red (Motor A) | OUT1 |
| Black (Motor A) | OUT2 |

### Summary

✅ Use the 12V input for motor power.
✅ Keep the 5V jumper ON if using 9V-12V input.
✅ Remove the jumper if you power ESP32 separately with a 5V supply.

Let me know if you need a KiCad schematic for this setup!

## Use L9110S motor drives

from machine import Pin, PWM

import time

# Define motor control pins

IN1 = Pin(18, Pin.OUT)  # Direction 1

IN2 = Pin(19, Pin.OUT)  # Direction 2

# Use PWM for speed control

speed_pwm = PWM(IN1, freq=1000)

def motor_forward(speed):

speed_pwm.duty(speed)  # Set speed (0-1023)

IN1.value(1)

IN2.value(0)

def motor_backward(speed):

speed_pwm.duty(speed)  # Set speed (0-1023)

IN1.value(0)

IN2.value(1)

def motor_stop():

speed_pwm.duty(0) # we need this to stop the motor

IN1.value(0)

IN2.value(0)

# Test motor

motor_forward(512)  # 50% speed forward

time.sleep(2)

motor_backward(512)  # 50% speed backward

time.sleep(2)

motor_stop()  # Stop motor

## Read the Encoders (Changed from what chatGPT suggested)

For reading the encoders we use the following wires:

| 3 | red | E1/1 | GND |
| --- | --- | --- | --- |
| 4 | green | E1/2 | 3.3V |
| 5 | yellow | E1/3 | quadrature encoder A |
| 6 | blue | E1/4 | quadrature encoder B |

Connection to ESP32 (C3 Zero Used)

| EV3 Wire | Function | ESP32 Pin (C3Zero) |
| --- | --- | --- |
| Blue | Encoder A (Signal) | GPIO 4 (or any input) |
| Yellow | Encoder B (Signal) | GPIO 5 (or any input) |
| Red | Encoder Ground | GND |
| Green | Encoder Power | 3.3V or 5V |

from machine import Pin import time # Define Encoder Pins ENC_A = Pin(4, Pin.IN, Pin.PULL_UP) # Encoder A ENC_B = Pin(5, Pin.IN, Pin.PULL_UP) # Encoder B encoder_count = 0 # Tracks motor position def encoder_callback(pin): global encoder_count if ENC_B.value() == 1: # Check B signal to determine direction encoder_count += 1 # Forward else: encoder_count -= 1 # Reverse # Attach interrupt to read encoder pulses ENC_A.irq(trigger=Pin.IRQ_RISING, handler=encoder_callback) # Main loop while True: print("Encoder Position:", encoder_count) time.sleep(0.1)

## Reference Links

https://lechnology.com/2017/03/using-lego-mindstorms-motors-with-beaglebone-blue/

https://en.depfields.com/dcmotor-pwmcontrol/